By 
Swapnil Acharya

Simulation of dynamic branch prediction scheme

Simulation of 2bit and 3bit dynamic branch prediction schemes

Table of Contents

OBJECTIVE	2

PROJECT REQUIREMENTS	2

DESCRIPTION OF BASE ALGORITHM: MERGE SORT	4

INPUT DATA FOR THE MERGE SORT ALGORITHM	4

DEMONSTRATION OF BASE MERGE SORT ALGORITHM	5

DESCRIPTION OF 2-BIT PREDICTOR SCHEME	6

OUTPUT OF 2-BIT PREDICTION SCHEME	10

DESCRIPTION OF 3-BIT PREDICTOR SCHEME	12

OUTPUT OF 3-BIT PREDICTION SCHEME	15

RESULTS	17

DISCUSSIONS	20

SUMMARY	20

REFERENCES	21

RUNNING THE SOURCE FILES	21

SOURCE CODE LISTINGS	22

# OBJECTIVE

The object of this project is to implement an algorithm and simulate the use of dynamic branch prediction schemes. Correct functioning of both the base algorithm and the dynamic branch prediction schemes are to be demonstrated.

# PROJECT REQUIREMENTS

- Implement one of the following types of algorithms:

  - Search

  - Solution of a set of linear equations

  - a fast sort method (Note: selection sort and insertion sort methods are not acceptable).

The main requirements for your selected algorithm are that it result in a minimum

of 50 lines of code and there are at least 3 distinct conditional branch statements

within it, one of these branch statements should be dependent on data associated

with the problem.

This assignment is to be done in C/C++. You are to substitute if-statements and goto-statements for higher level conditional constructs such as: for, while, do-while and

switch. This is so that the use of conditional branches will be obvious in your solution.

- Create a version of your code in which you simulate a 2-bit dynamic branch prediction scheme. For each branch keep track of the following statistics:

  - number of taken branches

  - number of not-taken branches

  - number of predictions

  - number of miss-predictions

In addition, you may need additional statistics for each branch in order to

implement the prediction scheme.

- Create another version of your code in which you simulate a (3,1) predictor scheme. Keep track of the same four required statistics in addition to any additional statistics for each branch in order to implement the prediction scheme.

- Execute the base algorithm along with each prediction scheme. Use a problem size that is large enough so that the number of predictions per branch are greater than 10; for inner loop branches this number should be on the order of 100. You may include the data for the base algorithm directly in the data section of the program instead of inputting it interactively or via a file.

- In your report, compare the two prediction schemes and show which was more accurate.

Turn in a summary report that includes the following materials:

- a written description that describes your base algorithm and predicts what the output of your base algorithm should be when it is applied to your supplied data.

- screen shots that demonstrate the base algorithm works.

- screen shots that demonstrate the dynamic branch prediction schemes work.

- describe your branch prediction schemes.

- for each branch in the program, identify the branch and display the corresponding statistics (see item 2 above). Comment on the results.

- a listing (source code) of your programs.

- the names of the team members are to be included on the first page of the report.

# DESCRIPTION OF BASE ALGORITHM: MERGE SORT

The base algorithm used in this project is Merge Sort. Merge Sort is based on dive and conquer approach. Merge sort partitions an input array into two halves, then recursively calls itself for the two partitions, and then merge the two sort partitions. The merge() function is used for sorting and merging the portioned halves at each step. Since this project requires simulation of branch prediction scheme. For loops and while loops were implemented via goto and if statement. The merge sort algorithm used here has 8 branches. Among 8 branches, there are two branches that depend on data associated with the problem (input array to be sorted).

# INPUT DATA FOR THE MERGE SORT ALGORITHM

A program (gen_numbers.cpp) that generates a text file that consists of randomly generated numbers was created. The contents of generated txt are as follows:

42 95 77 21 93 29 17 70 38 21 54 63 93 26 18 90 77 5 29 8 5 66 8 26 25 15 76 23 50 70 68 91 65 43 11 56 69 27 24 6 45 77 68 39 3 85 27 79 87 55 84 92 19 91 16 43 7 91 63 54 59 29 44 22 71 54 78 39 78 2 44 24 77 12 60 79 97 86 57 84 40 41 74 58 31 90 99 37 79 60 88 39 89 32 58 58 85 36 96 64 36 40 85 13 52 45 89 47 29 46 29 68 85 2 25 16 91 24 50 70 83 38 7 70 70 63 29 53 96 25 15 32 65 99 42 15 42 30 59 71 75 85 37 58 86 61 73 77 82 21 46 66 59 50 34 27 13 62 79 10 85 94 39 50 91 79 62 34 9 21 5 81 4 39 40 90 1 11 67 82 32 11 46 88 59 80 13 71 40 89 78 25 81 16 72 73 94 34 5 3 52 9 83 56 45 21 46 45 32 11 26 61 20 69 49 78 47 59 47 87 48 24 10 30 39 82 1 33 14 5 33 66 11 14 20 55 35 63 98 66 71 24 27 90 90 73 66 38 32 14 23 77 37 32 7 75 14 5 6 26 9 36 91 19 49 9 72 83 71 70 49 40 91 73 29 81 44 94 17 73 8 39 51 42 69 55 15 80 60 18 6 66 53 94 83 3 3 55 83 71

# DEMONSTRATION OF BASE MERGE SORT ALGORITHM

The implementation of base merge sort algorithm is in MergeSort.cpp. Figure 1 sows the output of this base merge sort implementation.

Figure 1: Output of Base Merge Sort Algorithm

# DESCRIPTION OF 2-BIT PREDICTOR SCHEME

The state diagram in Figure 2, shows two-bit prediction scheme. This state diagram shows that in STATE_00 the prediction by the 2-bit predictor is NOT TAKEN. If the program action was NOT TAKEN then the predictor stays in same state else if the program action was TAKEN then the predictor moves to STATE_01. The predictions and transitions for all states are done in similar manner as discussed here and shown by Figure 2.

Figure 2: The 2 Bit prediction scheme sate diagram [1]

The algorithm for 2-bit predictor scheme keeps track of some parameters for all 8 branches in the merge sort algorithm. The default state for all 8 branches are initialized to STATE_00. Figure 3 shows the parameters and a default constructor that initializes all the predictors to STATE_00. An array of objects (length  = number of branches) for class in Figure 3 is instantiated as global variable in the merge sort algorithm and individual array components are place as needed in the vicinity of the branches in the merge sort algorithm.

Figure 3: The parameters and default constructor for 2-bit prediction scheme.

Figure 4: Code Snippet of how and where the Branch statistics are computed.

Consider code snippet in Figure 4, when the if statement becomes true then branch is considered to be TAKEN. When a branch is TAKEN the number of taken branches is increased. Then to compute predictions and state updates, the program action, TAKEN in this case is passed to the function in Figure 5.

Figure 5, computation of state updates and predictions.

Initially the default state for all branches are set to STATE_00. For this case consider, the current state to be STATE_11.  As depicted by the state diagram in Figure 2, For STATE_11, the prediction by the 2-bit predictor is TAKEN. If the program action was TAKEN and the prediction by the predictor was also TAKEN, then the number of correct predictions is increased, and the state transition is performed.

# OUTPUT OF 2-BIT PREDICTION SCHEME

# DESCRIPTION OF 3-BIT PREDICTOR SCHEME

The algorithm for 3-bit predictor scheme keeps track of some parameters for all 8 branches in the merge sort algorithm like the 2-bit predictor. The default state for all 8 branches are initialized to STATE_00. Figure 6 shows the parameters that are sued to keep track of branch statistics.

Unlike the 2-bit predictor scheme, 3-bit predictor store history of branches. If the last 3 breaches were TAKEN then the history bits are set to TAKEN, TAKEN, TAKEN. A combination table of history bits is made, of ra 3bit predictor there are total of 8 combinations. These combinates are represented by integer values as shown in Figure 6.  The combination table is represented by a has table where the key is the representation of a combination and value is the prediction of that combination. The global history is maintained using a vector, the entries in this vector will be the representation of combinations. The default constructor initializes all the combinations in combination table as NOT TAKEN. In addition to that, the default constructor also adds “COMB_000” in the global history, i.e. history bits being NOT TAKEN, NOT TAKEN, NOT TAKEN.

Consider code snippet in Figure 4, when the if statement becomes true then branch is considered to be TAKEN. When a branch is TAKEN the number of taken branches is increased. Then to compute predictions and state updates, the program action, TAKEN in this case is passed to the function in Figure 7.

Initially all the entries in combinations are initialized to NOT TAKEN. For this case consider, the last entry in global history to be COMB_000, i.e. history bits being NOT TAKEN, NOT TAKEN, NOT TAKEN. The prediction entry for this history is fetched from combination table. If the prediction entry was TAKEN and the program action was NOT TAKEN then the number of incorrect predictions are update, the entry in the combination table for COMB_000 is update from TAKEN to NOT TAKEN and a new combination COMB_001 i.e. NOT TAKEN, NOT TAKEN, TAKEN is added to global history table.

In general, a combination table consisting of possible combination for 3 history bits is created. This is then initialized to either TAKEN or NOT TAKEN. Then an entry is added to global history table depending upon how the combination table was initialized. When the merge sort algorithm runs, depending upon the last entry in global history table and program action, the prediction count updates are done and entries in combination table are updated and an a new entry is made on global history table. The entries on global history table are simply left shifted, where the history bit 3’s entry becomes history bit 2’s entry,  history bit 2’s entry becomes history bit 1’s entry and history bit 3’s entry becomes the program action.

Figure 6: The parameters and default constructor for 3-bit prediction scheme.

Figure 7, updates of predictions count, combination table and global history table

# OUTPUT OF 3-BIT PREDICTION SCHEME

# RESULTS

| Branch | Statistics | 2-Bit Predictor | 3-Bit Predictor |
| --- | --- | --- | --- |
| 0 | Number of Taken Branches | 200 | 200 |
|  | Number of Not Taken Branches | 199 | 199 |
|  | Number of Correct Branch Predictions | 137 | 272 |
|  | Number of Miss Branch Predictions | 262 | 127 |
|  | Correct Branch Prediction | 34.34% | 68.17% |
|  | Miss Branch Prediction | 54.66% | 31.83% |
| 1 | Number of Taken Branches | 199 | 199 |
|  | Number of Not Taken Branches | 812 | 812 |
|  | Number of Correct Branch Predictions | 812 | 742 |
|  | Number of Miss Branch Predictions | 199 | 269 |
|  | Correct Branch Prediction | 80.32% | 73.39% |
|  | Miss Branch Prediction | 19.68% | 26.61% |
| 2 | Number of Taken Branches | 199 | 199 |
|  | Number of Not Taken Branches | 732 | 732 |
|  | Number of Correct Branch Predictions | 732 | 772 |
|  | Number of Miss Branch Predictions | 199 | 159 |
|  | Correct Branch Prediction | 78.63% | 82.92% |
|  | Miss Branch Prediction | 21.37% | 17.08% |
| 3 | Number of Taken Branches | 199 | 199 |
|  | Number of Not Taken Branches | 1281 | 1281 |
|  | Number of Correct Branch Predictions | 1281 | 1193 |
|  | Number of Miss Branch Predictions | 199 | 287 |
|  | Correct Branch Prediction | 86.55% | 80.61% |
|  | Miss Branch Prediction | 13.45% | 19.39% |
| 4 (Data dependent) | Number of Taken Branches | 650 | 650 |
|  | Number of Not Taken Branches | 631 | 631 |
|  | Number of Correct Branch Predictions | 596 | 670 |
|  | Number of Miss Branch Predictions | 685 | 611 |
|  | Correct Branch Prediction | 46.53% | 52.30% |
|  | Miss Branch Prediction | 53.47% | 47.70% |
| 5 (Data dependent) | Number of Taken Branches | 631 | 631 |
|  | Number of Not Taken Branches | 0 | 0 |
|  | Number of Correct Branch Predictions | 629 | 629 |
|  | Number of Miss Branch Predictions | 2 | 2 |
|  | Correct Branch Prediction | 99.68% | 99.68% |
|  | Miss Branch Prediction | 0.32% | 0.32% |
| 6 | Number of Taken Branches | 199 | 199 |
|  | Number of Not Taken Branches | 162 | 162 |
|  | Number of Correct Branch Predictions | 164 | 189 |
|  | Number of Miss Branch Predictions | 197 | 172 |
|  | Correct Branch Prediction | 45.43% | 52.35% |
|  | Miss Branch Prediction | 54.57% | 47.65% |
| 7 | Number of Taken Branches | 199 | 199 |
|  | Number of Not Taken Branches | 101 | 101 |
|  | Number of Correct Branch Predictions | 184 | 177 |
|  | Number of Miss Branch Predictions | 116 | 123 |
|  | Correct Branch Prediction | 61.33% | 59.00% |
|  | Miss Branch Prediction | 38.67% | 41.00% |

Table 1: Comparison of the performance of 2-bit and 3-bit predictor schemes.

Figure 8: Comparison of Correct Branch Prediction Percentage of both 2-bit and 3-bit predictors.

# DISCUSSIONS

The results in table 1 and figure 8 are for a problem size N = 200. Table 1 shows that, for the outer loop branches number of predictions per branches are higher 10 and for inner loop branches the number of predictions are in the order of hundred. Branch 0,1,2,3,6,7 are not data dependent branches where are branch 4 and 5 are data dependent. From figure 8 for both data dependent branches the 3-bit predictor perform better than 2-bit predictor since 3-bit prediction’s miss prediction % is less when compared to that of 3-bit predictor. This is also true for data independent branches. The miss prediction % of 3-bit prediction is higher than 2-bit predictor in only 3 branches (branch 1,3 and 7) whereas the miss prediction % of 2-bit predictor is higher than 3-bit predictor in 4 branches. (branch 0,2,4,6). The branches where 2-bit predictor has higher miss prediction than 3-bit predictor is significantly higher. Therefore, based on these results the 3-bit predictor performs slightly better than 2-bit predictor.

# SUMMARY

Merge sort algorithm was implemented in C++. The loops in the merge sort code were implemented using if statements and goto statements in order to simulate assembly code. 2-bit and 3-bit dynamic branch predictor schemes were implemented and integrated with the merge sort algorithm. The results indicated that 3-bit predictor scheme performed slightly better than 2-bit predictor scheme.

# REFERENCES

[1] John L. Hennessy and David A. Patterson, Computer Architecture A Quantitative Approach, Elsevier Inc., Sixth Edition, 2019, ISBN: 978-0-12-811905-1.

# RUNNING THE SOURCE FILES

- In a Unix/Linux terminal navigate to where the source files are stored

- Type make in terminal to trigger the makefile to run.

- The makefile will have created 3 executables

- To run base merge sort type ./msort in terminal

- To run merge sort with 2-bit predictor simulation type ./m2bit in terminal

- To run merge sort with 3-bit predictor simulation type ./m3bit in terminal

# SOURCE CODE LISTINGS

PROGRAM USED TO GENERATE RANDOM NUMERS IN A FILE

- /*

- *Date: 11/22/2020

- *File: gen_numbers.cpp

- *Description: This program generates random numbers and writes them in a file.

- *             The numbers of random numbers to be generated must be supplied as argument

- *             to the executable

- */

- #include <stdio.h>

- #include <stdlib.h>

- #include <time.h>

- /*

- * This

- *@param min the minimum value for the random number generation

- *@param max the maximum value for the random bumber generation

- *@returns a dynamic integer array

- */

- int main(int argc, char * argv[]){

- if(argc < 2){

- printf("Specify The numbers of random numbers to be generated \n");

- return 0;

- }

- int N = atoi(argv[1]);

- FILE *fp;

- fp = fopen("rand_numbers.txt","w+");

- //intialize random seed

- srand(time(NULL));

- //generate random number between

- for(int i = 0; i < N; i++){

- int final_rand_num = (rand()% (100-1)) + 1;

- fprintf(fp,"%d ",final_rand_num);

- }

- fclose(fp);

- }

MERGE SORT BASE ALGORITHM

- /*

- *Date: 11/21/2020

- *File: MergeSort.cpp

- */

- // C++ program for Merge Sort

- #include <stdlib.h>

- #include <stdio.h>

- using namespace std;

- //Problem size

- const int N = 200;

- /*

- *This function displays the contents of a dynamic array.

- *@param *ar the dynamic integer array whose contents are to be displayed

- *@param ar_size the number of elements inside the array or the lengh of the array

- *@returns None

- */

- void display_array_contents(int * ar, int ar_size){

- //initialize count

- int count = 0;

- display_loop:

- if (count > ar_size - 1){

- printf("\n");

- return;

- }

- printf("%d ", *(ar + count) );

- count = count + 1;

- goto display_loop;

- }

- // Merges two subarrays of arr[].

- // First subarray is arr[l..m]

- // Second subarray is arr[m+1..r]

- void merge(int *array, int l, int m, int r)

- {

- int i , j , k, nl, nr;

- //size of left sub-arrays

- nl = m-l+1;

- //size of right sub-arrays

- nr = r-m;

- // Create temp arrays

- int larr[nl];

- int rarr[nr];

- //copy to left temp array

- i = 0;

- for_loop_left:

- if( i > nl-1){ //BRANCH 1

- goto done_for_loop_left;

- }

- larr[i] = array[l + i];

- i++;

- goto for_loop_left;

- done_for_loop_left:

- j = 0;

- //copy to right temp array

- for_loop_right:

- if( j > nr-1){ //BRANCH 2

- goto done_for_loop_right;

- }

- rarr[j] = array[m + 1 + j];

- j++;

- goto for_loop_right;

- done_for_loop_right:

- // Merge the temp arrays back into arr[l..r]

- // Initial index of first subarray

- i = 0;

- // Initial index of second subarray

- j = 0;

- // Initial index of merged subarray

- k = l;

- //merge arrays

- merge_array_while_loop:

- //converting while to if, took 2 hours but nedded to change logic from && to ||

- if((i > nl-1) || (j > nr-1)){ //BRANCH 3

- goto done_merge_array_while_loop;

- }

- if(larr[i] <= rarr[j]){ //BRANCH 4, data dependent branching

- array[k] = larr[i];

- i++;

- k++;

- goto merge_array_while_loop;

- }

- if(larr[i] > rarr[j]){ //BRANCH 5, data dependent branching

- array[k] = rarr[j];

- j++;

- k++;

- goto merge_array_while_loop;

- }

- done_merge_array_while_loop:

- // Copy the remaining elements of

- // L[], if there are any

- copy_remaining_left_while_loop:

- if(i > nl-1){ //BRANCH 6

- goto done_copy_remaining_left_while_loop;

- }

- array[k] = larr[i];

- i++;

- k++;

- goto copy_remaining_left_while_loop;

- done_copy_remaining_left_while_loop:

- // Copy the remaining elements of

- // R[], if there are any

- copy_remaining_right_while_loop:

- if (j > nr-1) { //BRANCH 7

- goto done_copy_remaining_right_while_loop;

- }

- array[k] = rarr[j];

- j++;

- k++;

- goto copy_remaining_right_while_loop;

- done_copy_remaining_right_while_loop:

- return;

- }

- /*

- * This function sort an input array based on merge sort Algorithm.

- * Merge Sort is based on dive and conquer approach. Merge sort

- * partitions an input array into two halves, then recursively calls

- * itself for the two partitions, and then merge the two sort partitions.

- * @param array the input integer array that is to be sorted

- * @param l the left or the lower index of the array or the section of the

- *  array to be sorted

- * @param r the right or the upper index of the array or the section of the

- *  array to be sorted

- * @returns None the input array is sorted when the function is complete

- */

- void mergeSort(int *array, int l, int r)

- {

- int m;

- if(l > r - 1){ //BRANCH 0

- goto done_merge_sort;

- }

- //get the middle index for array partition

- // Same as (l+r)/2, but avoids overflow for large l and h

- m = l + (r-l)/2;

- //Recursively call the firt half of the array for sorting

- mergeSort(array, l, m);

- //Recursively call the firt half of the array for sorting

- mergeSort(array, m + 1, r);

- //Merge the sorted halves

- merge(array, l, m, r);

- done_merge_sort:

- //just a dummy bariabel so that the label "done_merge_sort" works

- int done = 0;

- }

- // Driver code

- int main()

- {

- printf("***********Simulation of Base Algorithms for Predictors **************\n\n");

- printf("***********Algorithm Used: Merge Sort*******************************\n\n");

- printf("**********Reading Numbers From File*********************************\n\n");

- FILE *fp;

- fp = fopen("rand_numbers.txt","r");

- int *arr = new int[N];

- for(int i = 0; i < N ; i++){

- fscanf(fp,"%d",(arr+i));

- }

- printf("***************Generated Array of Random Numbers ******************\n");

- display_array_contents(arr, N);

- printf("\n");

- printf("**************Started Sorting*************************************\n\n");

- mergeSort(arr, 0, N - 1);

- printf("**************Sorted Array is*************\n\n");

- display_array_contents(arr, N);

- printf("\n");

- return 0;

- }

2-BIT PREDICTOR IMPLEMENTATION

- /*

- * Date: 11/21/2020

- * File: BranchStats_2Bits.h

- * Description: The BranchStats_2Bits class in this file implements 2-bit prediction scheme

- */

- #ifndef _BRANCH_STATS_2_BITS_H_

- #define _BRANCH_STATS_2_BITS_H_

- #include <stdlib.h>

- #include <stdio.h>

- //representations for program actions and predictions

- const int NOT_TAKEN = 0;

- const int TAKEN = 1;

- //representations for states

- const int STATE_00 = 1000;

- const int STATE_01 = 2000;

- const int STATE_10 = 3000;

- const int STATE_11 = 4000;

- class BranchStats_2Bits{

- private:

- //variable to hold number of taken branches

- int number_of_taken_branches;

- //variable to hold number of not taken branches

- int number_of_not_taken_branches;

- //variable to hold branch predictor state for this particular branch

- int branch_predictor_state;

- //variable to hold number of correct predictions

- int number_of_correct_predictions;

- //variable to hold number of incorrect predictions

- int number_of_miss_predictions;

- public:

- //Default constructor, initializes 2-nit predictor to STATE_00

- BranchStats_2Bits(){

- this->number_of_taken_branches = 0;

- this->number_of_not_taken_branches = 0;

- this->branch_predictor_state = STATE_00;

- this->number_of_correct_predictions = 0;

- this->number_of_miss_predictions = 0;

- }

- //Overloaded constructor, initializes 2-nit predictor to supplied state as argument

- BranchStats_2Bits(int state){

- this->number_of_taken_branches = 0;

- this->number_of_not_taken_branches = 0;

- this->branch_predictor_state = state;

- this->number_of_correct_predictions = 0;

- this->number_of_miss_predictions = 0;

- }

- //method to increase number of taken branches

- void increase_num_taken_branches(){

- this->number_of_taken_branches++;

- }

- //method to fetch  number of taken branches

- int get_num_taken_branches() const{

- return this->number_of_taken_branches;

- }

- //method to increase number of not taken branches

- void increase_num_not_taken_branches(){

- this->number_of_not_taken_branches++;

- }

- //method to fetch  number of not taken branches

- int get_num_not_taken_branches() const{

- return this->number_of_not_taken_branches;

- }

- //method to fetch  number of correct branch predictions

- int get_num_correct_predictions() const{

- return this->number_of_correct_predictions;

- }

- //method to fetch  number of incorrect branch predictions

- int get_num_miss_predictions() const{

- return this->number_of_miss_predictions;

- }

- //method to display statis for this branch

- void print_statistics(){

- printf("Number of Taken Branches: %d\n",this->number_of_taken_branches);

- printf("Number of Not Taken Branches: %d\n",this->number_of_not_taken_branches);

- printf("Number of Correct Branch Predictions: %d\n",this->number_of_correct_predictions);

- printf("Number of Miss Branch Predictions: %d\n",this->number_of_miss_predictions);

- double total_predictions = this->number_of_correct_predictions + this->number_of_miss_predictions;

- double cpr = this->number_of_correct_predictions / total_predictions;

- double mpr = this->number_of_miss_predictions / total_predictions;

- printf("Correct Branch Prediction : %0.2f %%\n", cpr*100);

- printf("Miss Branch Prediction : %0.2f %% \n", mpr*100);

- }

- /*

- * This method depending upon current stae of the 2-bit predictor and

- * action taken by the program , update number of correct.incorrect predictions and

- * state transitions for the 2-bit preditor

- * @param program_action the action trak ny program i.e. if the branch was TAKEN or NOT TAKEN

- */

- void update_predictions(int program_action){

- //update predictions if it was correct or not correct

- if(this->branch_predictor_state == STATE_11){

- //STATE 11 Predicts TAKEN

- if(program_action == TAKEN){ //if program action was taken

- this->number_of_correct_predictions++;

- //state stays the same

- }

- else{ //if program action was not taken

- this->number_of_miss_predictions++;

- //update to STATE_10

- this->branch_predictor_state = STATE_10;

- }

- }

- else if(this->branch_predictor_state == STATE_10){

- //STATE 10 Predicts TAKEN

- if(program_action == TAKEN){ //if program action was taken

- this->number_of_correct_predictions++;

- this->branch_predictor_state = STATE_11;

- }

- else{ //if program action was not taken

- this->number_of_miss_predictions++;

- //update to STATE_00

- this->branch_predictor_state = STATE_00;

- }

- }

- else if(this->branch_predictor_state == STATE_00){

- //STATE 00 Predicts NOT TAKEN

- if(program_action == TAKEN){ //if program action was taken

- this->number_of_miss_predictions++;

- this->branch_predictor_state = STATE_01;

- }

- else{ //if program action was not taken

- this->number_of_correct_predictions++;

- //STATE DOES NOT CHANGE

- }

- }

- else{ //STATE_01

- //STATE 01 Predicts NOT TAKEN

- if(program_action == TAKEN){ //if program action was taken

- this->number_of_miss_predictions++;

- this->branch_predictor_state = STATE_11;

- }

- else{ //if program action was not taken

- this->number_of_correct_predictions++;

- //update to STATE_00

- this->branch_predictor_state = STATE_00;

- }

- }

- }

- };

- #endif

MERGE SORT WITH 2-BIT PREDICTOR INCLUDED

- /*

- *Date: 11/21/2020

- *File: MergeSort_2Bit.cpp

- */

- // C++ program for Merge Sort

- #include <stdlib.h>

- #include <stdio.h>

- #include <time.h>

- #include "BranchStats_2Bits.h"

- using namespace std;

- //Problem size

- const int N = 200;

- //Branch Prediction Parameters

- const int TOTAL_BRANCHES = 8;

- //declare a global stats

- BranchStats_2Bits branch_stats[TOTAL_BRANCHES];

- /*

- *This function displays the contents of a dynamic array.

- *@param *ar the dynamic integer array whose contents are to be displayed

- *@param ar_size the number of elements inside the array or the lengh of the array

- *@returns None

- */

- void display_array_contents(int * ar, int ar_size){

- //initialize count

- int count = 0;

- display_loop:

- if (count > ar_size - 1){

- printf("\n");

- return;

- }

- printf("%d ", *(ar + count) );

- count = count + 1;

- goto display_loop;

- }

- // Merges two subarrays of arr[].

- // First subarray is arr[l..m]

- // Second subarray is arr[m+1..r]

- void merge(int *array, int l, int m, int r)

- {

- int i , j , k, nl, nr;

- //size of left sub-arrays

- nl = m-l+1;

- //size of right sub-arrays

- nr = r-m;

- // Create temp arrays

- int larr[nl];

- int rarr[nr];

- //copy to left temp array

- i = 0;

- for_loop_left:

- if( i > nl-1){ //BRANCH 1

- //update taken branch stats

- branch_stats[1].increase_num_taken_branches();

- branch_stats[1].update_predictions(TAKEN);

- goto done_for_loop_left;

- }

- //update not taken branch stats

- branch_stats[1].increase_num_not_taken_branches();

- branch_stats[1].update_predictions(NOT_TAKEN);

- larr[i] = array[l + i];

- i++;

- goto for_loop_left;

- done_for_loop_left:

- j = 0;

- //copy to right temp array

- for_loop_right:

- if( j > nr-1){ //BRANCH 2

- //update taken branch stats

- branch_stats[2].increase_num_taken_branches();

- branch_stats[2].update_predictions(TAKEN);

- goto done_for_loop_right;

- }

- //update not taken branch stats

- branch_stats[2].increase_num_not_taken_branches();

- branch_stats[2].update_predictions(NOT_TAKEN);

- rarr[j] = array[m + 1 + j];

- j++;

- goto for_loop_right;

- done_for_loop_right:

- // Merge the temp arrays back into arr[l..r]

- // Initial index of first subarray

- i = 0;

- // Initial index of second subarray

- j = 0;

- // Initial index of merged subarray

- k = l;

- //merge arrays

- merge_array_while_loop:

- //converting while to if, took 2 hours but nedded to change logic from && to ||

- if((i > nl-1) || (j > nr-1)){ //BRANCH 3

- //update taken branch stats

- branch_stats[3].increase_num_taken_branches();

- branch_stats[3].update_predictions(TAKEN);

- goto done_merge_array_while_loop;

- }

- //update not taken branch stats

- branch_stats[3].increase_num_not_taken_branches();

- branch_stats[3].update_predictions(NOT_TAKEN);

- if(larr[i] <= rarr[j]){ //BRANCH 4, data dependent branching

- array[k] = larr[i];

- i++;

- k++;

- //update taken branch stats

- branch_stats[4].increase_num_taken_branches();

- branch_stats[4].update_predictions(TAKEN);

- goto merge_array_while_loop;

- }

- //update not taken branch stats

- branch_stats[4].increase_num_not_taken_branches();

- branch_stats[4].update_predictions(NOT_TAKEN);

- if(larr[i] > rarr[j]){ //BRANCH 5, data dependent branching

- array[k] = rarr[j];

- j++;

- k++;

- //update taken branch stats

- branch_stats[5].increase_num_taken_branches();

- branch_stats[5].update_predictions(TAKEN);

- goto merge_array_while_loop;

- }

- //update not taken branch stats

- branch_stats[5].increase_num_not_taken_branches();

- branch_stats[5].update_predictions(NOT_TAKEN);

- done_merge_array_while_loop:

- // Copy the remaining elements of

- // L[], if there are any

- copy_remaining_left_while_loop:

- if(i > nl-1){ //BRANCH 6

- //update taken branch stats

- branch_stats[6].increase_num_taken_branches();

- branch_stats[6].update_predictions(TAKEN);

- goto done_copy_remaining_left_while_loop;

- }

- //update not taken branch stats

- branch_stats[6].increase_num_not_taken_branches();

- branch_stats[6].update_predictions(NOT_TAKEN);

- array[k] = larr[i];

- i++;

- k++;

- goto copy_remaining_left_while_loop;

- done_copy_remaining_left_while_loop:

- // Copy the remaining elements of

- // R[], if there are any

- copy_remaining_right_while_loop:

- if (j > nr-1) { //BRANCH 7

- //update taken branch stats

- branch_stats[7].increase_num_taken_branches();

- branch_stats[7].update_predictions(TAKEN);

- goto done_copy_remaining_right_while_loop;

- }

- //update not taken branch stats

- branch_stats[7].increase_num_not_taken_branches();

- branch_stats[7].update_predictions(NOT_TAKEN);

- array[k] = rarr[j];

- j++;

- k++;

- goto copy_remaining_right_while_loop;

- done_copy_remaining_right_while_loop:

- return;

- }

- // l is for left index and r is

- // right index of the sub-array

- // of arr to be sorted

- void mergeSort(int *array, int l, int r)

- {

- int m;

- if(l > r - 1){ //BRANCH 0

- //update taken branch stats

- branch_stats[0].increase_num_taken_branches();

- branch_stats[0].update_predictions(TAKEN);

- goto done_merge_sort;

- }

- //update not taken branch stats

- branch_stats[0].increase_num_not_taken_branches();

- branch_stats[0].update_predictions(NOT_TAKEN);

- // Same as (l+r)/2, but avoids

- // overflow for large l and h

- //int m = (l + r - l) / 2;

- m = l + (r-l)/2;

- // Sort first and second halves

- mergeSort(array, l, m);

- mergeSort(array, m + 1, r);

- merge(array, l, m, r);

- done_merge_sort:

- int done = 0;

- }

- // Driver code

- int main()

- {

- printf("***********Simulation of 2 Bit Branch Prediction Scheme**************\n\n");

- printf("***********Algorithm Used: Merge Sort*******************************\n\n");

- printf("**********Reading Numbers From File*********************************\n\n");

- FILE *fp;

- fp = fopen("rand_numbers.txt","r");

- int *arr = new int[N];

- for(int i = 0; i < N ; i++){

- fscanf(fp,"%d",(arr+i));

- }

- printf("***************Generated Array of Random Numbers ******************\n");

- display_array_contents(arr, N);

- printf("\n");

- printf("**************Started Sorting*************************************\n\n");

- mergeSort(arr, 0, N - 1);

- printf("**************Sorted Array is*************\n\n");

- display_array_contents(arr, N);

- printf("\n");

- printf("*********Branch Statistics***********\n\n");

- for(int i = 0; i < TOTAL_BRANCHES; i++){

- printf("Branch: %d\n",i);

- branch_stats[i].print_statistics();

- printf("\n");

- }

- return 0;

- }

3-BIT PREDICTOR IMPLEMENTATION

- /*

- * Date: 11/21/2020

- * File: BranchStats_2Bits.h

- * Description: The BranchStats_3Bits class in this file implements 3-bit prediction scheme

- */

- #ifndef _BRANCH_STATS_3_BITS_H_

- #define _BRANCH_STATS_3_BITS_H_

- #include <stdlib.h>

- #include <stdio.h>

- #include <unordered_map>

- #include <vector>

- //representations for program actions and predictions

- const int NOT_TAKEN = 0;

- const int TAKEN = 1;

- //representations for possible hisotry bits

- const int COMB_000 = 1000; //N,N,N

- const int COMB_001 = 2000; //N,N,T

- const int COMB_010 = 3000; //N,T,N

- const int COMB_011 = 4000; //N,T,T

- const int COMB_100 = 5000; //T,N,N

- const int COMB_101 = 6000; //T,N,T

- const int COMB_110 = 7000; //T,T,N

- const int COMB_111 = 8000; //T,T,T

- class BranchStats_3Bits{

- private:

- //variable to hold number of taken branches

- int number_of_taken_branches;

- //variable to hold number of not taken branches

- int number_of_not_taken_branches;

- //variable to hold number of correct predictions

- int number_of_correct_predictions;

- //variable to hold number of incorrect predictions

- int number_of_miss_predictions;

- //a hash table to hold combination tabel of hisotry bits and current predict value

- //with combination being the key and prediction value being the value for the hash table

- std::unordered_map<int,int> combination_table;

- //a vector to hold the global history bits

- std::vector<int> global_history_table;

- public:

- //default constructor initializes all combination table for 3 bt

- //predictor to be NOT TAKEN

- BranchStats_3Bits(){

- this->number_of_taken_branches = 0;

- this->number_of_not_taken_branches = 0;

- this->number_of_correct_predictions = 0;

- this->number_of_miss_predictions = 0;

- //initialize combination tabel for 3 bit predictor

- for(int i=1000; i < 9000; i+= 1000){

- this->combination_table[i] = NOT_TAKEN;

- }

- //initialize global history table with T,T,T

- this->global_history_table.push_back(COMB_111);

- }

- //method to increase number of taken branches

- void increase_num_taken_branches(){

- this->number_of_taken_branches++;

- }

- //method to fetch  number of taken branches

- int get_num_taken_branches() const{

- return this->number_of_taken_branches;

- }

- //method to increase number of not taken branches

- void increase_num_not_taken_branches(){

- this->number_of_not_taken_branches++;

- }

- //method to fetch  number of not taken branches

- int get_num_not_taken_branches() const{

- return this->number_of_not_taken_branches;

- }

- //method to fetch  number of correct branch predictions

- int get_num_correct_predictions() const{

- return this->number_of_correct_predictions;

- }

- //method to fetch  number of incorrect branch predictions

- int get_num_miss_predictions() const{

- return this->number_of_miss_predictions;

- }

- //method to display statis for this branch

- void print_statistics(){

- printf("Number of Taken Branches: %d\n",this->number_of_taken_branches);

- printf("Number of Not Taken Branches: %d\n",this->number_of_not_taken_branches);

- printf("Number of Correct Branch Predictions: %d\n",this->number_of_correct_predictions);

- printf("Number of Miss Branch Predictions: %d\n",this->number_of_miss_predictions);

- double total_predictions = this->number_of_correct_predictions + this->number_of_miss_predictions;

- double cpr = this->number_of_correct_predictions / total_predictions;

- double mpr = this->number_of_miss_predictions / total_predictions;

- printf("Correct Branch Prediction : %0.2f %%\n", cpr*100);

- printf("Miss Branch Prediction : %0.2f %% \n", mpr*100);

- }

- /*

- *This method gets the last entry from global history tabel and goes to the combination table,

- * and fetches the prediction for that entry. Then the prediction and the action taken by parameters

- * are use to update correct/incorrect predictions, update the entry on combination table and add

- * a new entry in global history.

- */

- void update_predictions(int program_action){

- //get last item from history table

- int last_history = this->global_history_table.back();

- //get combination

- if(last_history == COMB_000){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_000];;

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_001);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_000);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_000] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_000);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_000]= TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_001);

- }

- }

- //get combination

- else if(last_history == COMB_001){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_001];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_011);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_010);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_001] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_010);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_001] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_011);

- }

- }

- //get combination

- else if(last_history == COMB_010){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_010];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_101);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_100);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_010] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_100);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_010] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_101);

- }

- }

- //get combination

- else if(last_history == COMB_011){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_011];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_111);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_110);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_011] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_110);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_011] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_111);

- }

- }

- //get combination

- else if(last_history == COMB_100){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_100];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_001);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_000);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_100] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_000);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_100] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_001);

- }

- }

- //get combination

- else if(last_history == COMB_101){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_101];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_011);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_010);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_101] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_010);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_101] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_011);

- }

- }

- //get combination

- else if(last_history == COMB_110){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_110];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_101);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_100);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_110] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_100);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_110] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_101);

- }

- }

- //get combination

- else if(last_history == COMB_111){

- //go to combination table to get prediction

- int prediction = this->combination_table[COMB_111];

- //if prediction and program action were taken

- if((prediction == TAKEN) && (program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_011);

- }

- //if prediction and program action were not taken

- else if( (prediction == NOT_TAKEN) && (program_action == NOT_TAKEN)){

- //update number of correct predictions

- this->number_of_correct_predictions++;

- //update history table

- this->global_history_table.push_back(COMB_010);

- }

- else if( (prediction == TAKEN ) && (program_action == NOT_TAKEN) ){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_111] = NOT_TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_010);

- }

- else if( ( prediction == NOT_TAKEN) && ( program_action == TAKEN)){

- //update number of correct predictions

- this->number_of_miss_predictions++;

- //update combination_table

- this->combination_table[COMB_111] = TAKEN;

- //update history table

- this->global_history_table.push_back(COMB_011);

- }

- }

- }

- };

- #endif

MERGE SORT WITH 3-BIT PREDICTOR INCLUDED

- /*

- *Date: 11/21/2020

- *File: MergeSort_3Bit.cpp

- */

- // C++ program for Merge Sort

- #include <stdlib.h>

- #include <stdio.h>

- #include <time.h>

- #include "BranchStats_3Bits.h"

- using namespace std;

- //Problem size

- const int N = 200;

- //Branch Prediction Parameters

- const int TOTAL_BRANCHES = 8;

- //declare a global stats

- BranchStats_3Bits branch_stats[TOTAL_BRANCHES];

- /*

- *This function displays the contents of a dynamic array.

- *@param *ar the dynamic integer array whose contents are to be displayed

- *@param ar_size the number of elements inside the array or the lengh of the array

- *@returns None

- */

- void display_array_contents(int * ar, int ar_size){

- //initialize count

- int count = 0;

- display_loop:

- if (count > ar_size - 1){

- printf("\n");

- return;

- }

- printf("%d ", *(ar + count) );

- count = count + 1;

- goto display_loop;

- }

- // Merges two subarrays of arr[].

- // First subarray is arr[l..m]

- // Second subarray is arr[m+1..r]

- void merge(int *array, int l, int m, int r)

- {

- int i , j , k, nl, nr;

- //size of left sub-arrays

- nl = m-l+1;

- //size of right sub-arrays

- nr = r-m;

- // Create temp arrays

- int larr[nl];

- int rarr[nr];

- //copy to left temp array

- i = 0;

- for_loop_left:

- if( i > nl-1){ //BRANCH 1

- //update taken branch stats

- branch_stats[1].increase_num_taken_branches();

- branch_stats[1].update_predictions(TAKEN);

- goto done_for_loop_left;

- }

- //update not taken branch stats

- branch_stats[1].increase_num_not_taken_branches();

- branch_stats[1].update_predictions(NOT_TAKEN);

- larr[i] = array[l + i];

- i++;

- goto for_loop_left;

- done_for_loop_left:

- j = 0;

- //copy to right temp array

- for_loop_right:

- if( j > nr-1){ //BRANCH 2

- //update taken branch stats

- branch_stats[2].increase_num_taken_branches();

- branch_stats[2].update_predictions(TAKEN);

- goto done_for_loop_right;

- }

- //update not taken branch stats

- branch_stats[2].increase_num_not_taken_branches();

- branch_stats[2].update_predictions(NOT_TAKEN);

- rarr[j] = array[m + 1 + j];

- j++;

- goto for_loop_right;

- done_for_loop_right:

- // Merge the temp arrays back into arr[l..r]

- // Initial index of first subarray

- i = 0;

- // Initial index of second subarray

- j = 0;

- // Initial index of merged subarray

- k = l;

- //merge arrays

- merge_array_while_loop:

- //converting while to if, took 2 hours but nedded to change logic from && to ||

- if((i > nl-1) || (j > nr-1)){ //BRANCH 3

- //update taken branch stats

- branch_stats[3].increase_num_taken_branches();

- branch_stats[3].update_predictions(TAKEN);

- goto done_merge_array_while_loop;

- }

- //update not taken branch stats

- branch_stats[3].increase_num_not_taken_branches();

- branch_stats[3].update_predictions(NOT_TAKEN);

- if(larr[i] <= rarr[j]){ //BRANCH 4, data dependent branching

- array[k] = larr[i];

- i++;

- k++;

- //update taken branch stats

- branch_stats[4].increase_num_taken_branches();

- branch_stats[4].update_predictions(TAKEN);

- goto merge_array_while_loop;

- }

- //update not taken branch stats

- branch_stats[4].increase_num_not_taken_branches();

- branch_stats[4].update_predictions(NOT_TAKEN);

- if(larr[i] > rarr[j]){ //BRANCH 5, data dependent branching

- array[k] = rarr[j];

- j++;

- k++;

- //update taken branch stats

- branch_stats[5].increase_num_taken_branches();

- branch_stats[5].update_predictions(TAKEN);

- goto merge_array_while_loop;

- }

- //update not taken branch stats

- branch_stats[5].increase_num_not_taken_branches();

- branch_stats[5].update_predictions(NOT_TAKEN);

- done_merge_array_while_loop:

- // Copy the remaining elements of

- // L[], if there are any

- copy_remaining_left_while_loop:

- if(i > nl-1){ //BRANCH 6

- //update taken branch stats

- branch_stats[6].increase_num_taken_branches();

- branch_stats[6].update_predictions(TAKEN);

- goto done_copy_remaining_left_while_loop;

- }

- //update not taken branch stats

- branch_stats[6].increase_num_not_taken_branches();

- branch_stats[6].update_predictions(NOT_TAKEN);

- array[k] = larr[i];

- i++;

- k++;

- goto copy_remaining_left_while_loop;

- done_copy_remaining_left_while_loop:

- // Copy the remaining elements of

- // R[], if there are any

- copy_remaining_right_while_loop:

- if (j > nr-1) { //BRANCH 7

- //update taken branch stats

- branch_stats[7].increase_num_taken_branches();

- branch_stats[7].update_predictions(TAKEN);

- goto done_copy_remaining_right_while_loop;

- }

- //update not taken branch stats

- branch_stats[7].increase_num_not_taken_branches();

- branch_stats[7].update_predictions(NOT_TAKEN);

- array[k] = rarr[j];

- j++;

- k++;

- goto copy_remaining_right_while_loop;

- done_copy_remaining_right_while_loop:

- return;

- }

- // l is for left index and r is

- // right index of the sub-array

- // of arr to be sorted

- void mergeSort(int *array, int l, int r)

- {

- int m;

- if(l > r - 1){ //BRANCH 0

- //update taken branch stats

- branch_stats[0].increase_num_taken_branches();

- branch_stats[0].update_predictions(TAKEN);

- goto done_merge_sort;

- }

- //update not taken branch stats

- branch_stats[0].increase_num_not_taken_branches();

- branch_stats[0].update_predictions(NOT_TAKEN);

- // Same as (l+r)/2, but avoids

- // overflow for large l and h

- //int m = (l + r - l) / 2;

- m = l + (r-l)/2;

- // Sort first and second halves

- mergeSort(array, l, m);

- mergeSort(array, m + 1, r);

- merge(array, l, m, r);

- done_merge_sort:

- int done = 0;

- }

- // Driver code

- int main()

- {

- printf("***********Simulation of 3 Bit Branch Prediction Scheme**************\n\n");

- printf("***********Algorithm Used: Merge Sort*******************************\n\n");

- printf("**********Reading Numbers From File*********************************\n\n");

- FILE *fp;

- fp = fopen("rand_numbers.txt","r");

- int *arr = new int[N];

- for(int i = 0; i < N ; i++){

- fscanf(fp,"%d",(arr+i));

- }

- printf("***************Generated Array of Random Numbers ******************\n");

- display_array_contents(arr, N);

- printf("\n");

- printf("**************Started Sorting*************************************\n\n");

- mergeSort(arr, 0, N - 1);

- printf("**************Sorted Array is*************\n\n");

- display_array_contents(arr, N);

- printf("\n");

- printf("*********Branch Statistics***********\n\n");

- for(int i = 0; i < TOTAL_BRANCHES; i++){

- printf("Branch: %d\n",i);

- branch_stats[i].print_statistics();

- printf("\n");

- }

- return 0;

- }